M-HMDS

Metal nitrides are intensely investigated because they can offer high melting points, excellent corrosion resistance, high hardness, electronic and magnetic properties superior to the corresponding metals/metal oxides. Thus, they are used in multiple diverse applications including refractory materials, semiconductors, electronic devices and energy storage/conversion systems.

Common metal nitride syntheses use gas phase deposition or chemical routes. As mentioned for Novel SSEs studies, gas phase depositions methods generally require specialized and expensive equipment with relatively low production rates (generally <100 nm/min), limiting the ability to scale for industrial applications. In contrast, chemical routes are more versatile and allow production of a broader range of metal nitrides. However, it is difficult to achieve impurity-free products with optimal morphology control, and toxic precursors are often used.

Traditional chemical syntheses for metal nitrides involve high-temperature reactions (800-2000 °C) of parent metals, metal oxides, or other metal precursors with selected N sources, typically N2/NH3.

Here, simple, novel and scalable reactions of metal chlorides (MClx) with hexamethyldisilazane [HMDS, (Me3Si)2NH] in THF or ACN at low temperatures (ambient to 60 °C/N2) to produce metal nitride precursors, M(NHSiMe3)x (denoted as M-HMDS for convenience) were explored. For example, the synthesis of Al-HMDS precursor towards AlN is as follows.

The byproduct Me3SiCl (boiling point = 57 °C) can be removed easily along with THF/ACN and excess HMDS by drying at 80 °C/1 h/vacuum (vac). The dried Al-HMDS precursor can be further heated to 1600 °C/4 h/N2 in a tube furnace to produce AlN.

This work focuses primarily on the Al-HMDS precursor as well as produced AlN. It was further extended via proof of principle studies to other metal chloride systems, including Zn-HMDS, Cu-HMDS, Fe-HMDS and Bi-HMDS. These formed precursors are volatile, offering the potential utility as gas-phase deposition precursors for their corresponding metal nitrides.

Al-HMDS

Structures and molecular weights (MWs) of the synthesized the Al-HMDS precursor were analyzed by MALDI-ToF and MALDI-calculation. Overall, structures of the Al-HMDS precursor contain 2-4 units with Al-N bonds, suggesting oligomers and/or cyclomers with Al-N backbones. Some peaks also indicate -Cl inclusion and complexes with THF, consistent with NMR and EDX studies. Examples of predicted structures are as follows.

The synthesized Al-HMDS precursor was heated to different temperatures (800-1600 °C/4 h/N2). BET analyses show that Al-HMDS heated to 800° or 1200 °C show similar high specific surface areas (SSAs) >200 m2/g. On heating to 1600 °C, the resulting powders exhibit a much-reduced SSA of 28 m2/g coincident with grain growth. The high SSAs at 800° and 1200 °C indicate small pore sizes (4-5 nm) and average particle sizes (APSs) of ~20 nm.

XRD data for lower temperatures show primarily amorphous materials and only at 1600 °C are clear crystalline peaks for AlN observed, see below. When heated to 1600 °C/2 h/N2, a mixture of α-Al2O3 and wurtzite AlN phases are exhibited, and the AlN phase intensity increases with prolonged heating (1600 °C/4 h/N2). The presence of Al2O3 is likely due to moisture uptake as Al-HMDS shows high SSAs, making it highly susceptible to oxidation.

27Al MAS NMR spectra show that after drying at 80 °C/1 h/vac, Al is primarily present in [AlO6] units with a small amount of [AlO5] units. The [AlO5] peak intensity increased after heating to 800 °C/4 h/N2. After heating to 1600 °C/4 h/N2, a large peak at 114 ppm occurs, indicating AlN. Another peak at 14 ppm suggests [AlO6] units, but with different [AlO6] peak positions and narrower characteristics, indicating that the Al environment is more symmetrical for Al-HMDS heated to 1600 °C, likely a result of increased crystallinity, corresponding to XRD.

Overall, process optimization needs further attention to eliminate Al2O3; however, this approach is proven valid to produce high surface area AlN powder and should offer the potential for making coatings. The high surface areas are indicative of nanoscale particles. As a result, the heated Al-HMDS powders can also be used as additives to polymer matrices to modify their thermal stabilities, offering another potential application.

Polymer Matrix + Al-HMDS

Previously studied polymers, DGEBA-TMDS and DEO-OHS, were used as matrices, see the Oxysilylation page. Selected amounts (5, 10, and 25 wt.%) of Al-HMDS powders heated to 800 °C/4 h/N2 were added to the matrices.

DGEBA-TMDS + 0-25 wt.% Al-HMDS composites are all viscous liquids. In general, with Al-HMDS addition, decomposition temperatures (Td5%) increases slightly (increments of ~5 °C), except for 25 wt.% Al-HMDS, and ceramic yield (CY) increases with the Al-HMDS amount.

DEO-OHS + 0-25 wt.% Al-HMDS composites form black, flexible films. With 10 wt.% Al-HMDS, Td5% shows an ~100 °C increment, suggesting significant improvement in the polymer’s thermal stability. However, when the Al-HMDS content is further increased to 25 wt.%, both Td5% and CY decrease, lower than the pristine DEO-OHS. It is likely that the large amount of Al-HMDS impedes reaction between DEO and OHS as the catalyst likely binds preferentially to nitrogen, leaving considerable amounts of unreacted Si-H groups, resulting in less rigid structures with poorer thermal stabilities. This also explains the slight Td5% decrease for DGEBA-TMDS + 25 wt.% Al-HMDS.

In conclusion, flexible solid films with Al-HMDS (800 °C/4 h/N2) powders added to DEO-OHS were successfully cast, effectively modifying the thermal stabilities of polymer matrices with controlled loadings.

Other M-HMDS

Other metal chloride systems were also briefly explored including Zn-HMDS, Cu-HMDS, Fe-HMDS and Bi-HMDS, as potential precursors to corresponding metal nitrides. MALDI study indicates oligomers/cyclomers with M-N bonds, similar to Al-HMDS. TGA-DTA suggests that these M-HMDS precursors are volatile when heated (typically 500-700 °C), providing the potential to be used as gas-phase deposition precursors for their corresponding metal nitrides.

Examples of predicted structures of M-HMDS precursors:

Example thermal decomposition pathways for M-HMDS precursors:

For comprehensive analyses and discussions, please see the published paper:

Continue reading: